UGANDA INSTITUTE OF INFORMATION AND COMMUNICATIONS TECHNOLOGY

END OF SEMESTER II EXAMINATIONS

DEPARTMENT		:	ICT

YEAR			:	TWO

SEMESTER		:	TWO

PROGRAM		:	DIPLOMA IN COMPUTER SCIENCE

COURSE			:	COMPUTER WORKSHOP PRACTICE

COURSE CODE		:	ICT 121

DATE			:	SATURDAY, JUNE 29, 2024

TIME			:	09:00 AM – 12:00 NOON

DURATION		:	3 Hours

INSTRUCTIONS

- The paper consists of two Sections A and B

- Answer ALL questions in Section A and ANY THREE questions in Section B

- Credit will be given to relevant examples and illustrations

- Don Not Write anything on this question paper

SECTION A 1

- A combination of software and hardware is referred to as

A: prom	B: rom c: firmware	 D: bios memory	E: flash memory

- Which of the following might be associated with computer science?

- The study of data, data processing and information management

- The use of computer hardware, software and telecommunications devices to store, manipulate, convert, protect, send and receive data

- The study of both computer hardware and software design D: None of the above

- You have a computer that is running Windows 7, You want to upgrade this computer to Windows 11. Which of the following must you do first?

### A: Reformat the hard disk	B: Upgrade the system to windows 10	C: Backup the data	D: Start the upgrade right away

- Which of the following is not a type of printer?

A: Inkjet printer	B: Laser printer	C: Dot matrix	D: Laser jet printer

- Which of the following is the most current type of processor socket

A: ZIF		B: PGA		C: SVGA	D: DIP		E: UXVGA

- How many logical partitions are allowed on a single windows 10 drive

A: 1	B: 2	C:	4	D: 8	E: Unlimited

- A technician, wants to run antivirus scans on every machine on a daily basis. To minimize the impact to all system resources, which of the following is the BEST time to schedule the scans to run

A; 5:00 pm to 7:00pm	B: 7:30 am to 9:30am C: 11:00 pm to 1:00 am D: 12:00 pm to 2:00 pm

- A customer reports that a laser printer has a paper jam error at the output assembly. After a visual inspection, there are no visible paper jams. Which of the following are the MOST likely causes? (Select TWO)

A: Fuser flag sensor is defective	B: Transfer Roller is defective	C: Exit flag sensor is defective	D: Toner cartridge is defective 	E: Paper input sensor is defective

### When disassembling a laptop, a technician, should do which of the following?

### A: Remove the hard drive and all the data	B: Carefully place the screws in one container so that they do not get lost 	C: Refer to the manufacturer documentation	D: Power the laptop

- A user, would like 50GB of data transferred from his old computer to his new computer. Which of the following would be the BEST way for a technician, to transfer the user's data

A: Flash drive	B: DVD drive	C: Floppy drive	D: USB hard drive adaptor

- A user complains that the document smears on documents output from a laser printer. What is the most likely cause of the problem?

A: The ink cartridge is out of ink	B: Defective laser scanner	C: Defective fuser

D: Broken primary colour wire

- You're working on a computer and it fails to recognize a new SATA hard drive after the hard drive has been properly installed. What should be the first step to diagnose the issue?

A: Check the BIOS for proper drive settings	B: Replace the hard drive	C: Update the motherboard firmware	D: Replace the computer

- You are assembling a computer for a client and need to prevent ESD. What should you do first before touching any components?

A: Continuously touch metal to discharge static	B: Wear leather shoes		C: Wear an anti-static wrist strap	D: Wear jewellery on the wrist

- A user is reporting that their computer is running noticeably slower. Upon inspection, you find that the CPU is constantly at a high usage rate, even when the computer is idle. What is the best solution?

A: Upgrade the CPU to a higher-end model	B: Reinstall the Operating System	C: Check for malware or background programs D: Replace the power supply

- A technician needs to replace an HDD in a desktop PC. What should the technician do FIRST?

A: Perform disk check	B: Update firmware	C: Backup user data	D: Defragment HDD

- A light-sensitive device that converts the printed text, drawing, or other images into digital form is called ____.

A: Plotter	B: Printer	C: Scanner	D: OMR

- The resolution of Laser Printers is specified in terms of __?

A: CPM		B: PPM		C: DPI		D: LPM

- Which among the following is not a peripheral hardware device in a computer system?

A: Keyboard	B: Optical drive	C: Printer	D: HDD

- A microprocessor unit, a memory unit, and an input/output unit form a ____

A: Microcomputer	B: CPU		C: Compiler	D: ALU

20. USB refers to

- a storage device B: a processor C: a port type D: a serial bus standard

SECTION A2

- If Hard Disk is not detected then what is the cause and possible solution of that problem								[3 marks]

- Classify different types of printers based on technology		[3 marks]

- Identify the functions of a BIOS						[3 marks]

- List the main parts of a microprocessor				[3 marks]

- State any four hand tools found in a computer workshop		[4 marks]

- State any four common external causes of computer power supply failures 										[4 marks]

### SECTION B (EACH QUESTION CARRY 20 MARKS: CHOOSE THREE)

### Question one

### What is the purpose of power supply in computer systems												[2 marks]

### Classify the broad categories of power supply in computers	[4 marks]

### Explain why non-linear power supply is popular in computer systems											[6 marks]

### Assume you are the system administrator and of late you have realised that the mains (UMEME) power fluctuations are frequent and have led to the destruction of some your computers. Explain three ways in which you would protect your equipment against the Umeme power fluctuations. 										[8 marks]

### Question two

- Define electronic waste (e-waste)					[2 marks]

- State any three harmful human health effects caused by improper disposal of electronic device 							[3 marks]

- Identify three main drivers of e-waste generation			[6 marks]

- Describe any three strategies that are used in managing e-waste [9 marks]

### Question three

### State three major components that make up a computer system											[3 marks]

### Differentiate a BIOS from a CMOS					[4 marks]

### Outline four advantages of cache memory over other memories in a computer system 								[4 marks]

### In computer maintenance, why do you need utility software in computers										[5 marks]

### You have just bought a new sound card for your computer. Explain

### how your new sound card will work with the old computer. 	[4 marks]

### Question four

### What is a malware in computer system				[2 marks]

### Describe three basic principles of information security		[6 marks]

### Explain the four stages or phases a computer virus goes through to

### Infect and destroy a computer system					[8 marks]

- Classify each of the following as a violation of confidentiality, of integrity, of availability, or both							[4 marks]

- John copies Mary’s homework.

- Paul crashes Linda’s system.

- Carol changes the amount of Angelo’s check from $100 to $1,000

- Gina forges Roger’s signature on a deed.

### Question five

### Define the term media in computer application 			[2marks]

- Explain 2 broad classifications of network media							[4marks]

- Explain the term Crosstalk and give three ways it can be minimized											[6marks]

- With the aid of a well labeled diagram give three major applications of a coaxial cable in signal transmission 				[6marks]

- Define the term RF communication and briefly Explain how it works 											[2 marks]

END